VOLUME D11

GEOPHYSICAL INVESTIGATION

RESISTIVITY SURVEY

| Volume | Km from | Km to | Task | Length | Survey Type |
| --- | --- | --- | --- | --- | --- |
| D01 | 800 | 1+300 | i | 0.5 | Seismic |
| D02 | 27+200 | 27+500 | i | 0.3 | Seismic |
| D03 | 28+400 | 28+600 | i | 0.2 | Seismic |
| D04 | 28+600 | 29+600 | i | 1 | Resistivity |
| D05 | 36+800 | 37+300 | i | 0.5 | Resistivity |
| D06 | 37+300 | 37+800 | i | 0.5 | Seismic |
| D07 | 37+800 | 38+900 | i | 1.2 | Resistivity |
| D08 | 10+200 | 10+700 | iii LLR | 0.5 | Seismic |
| D09 | 300 | 900 | iii | 0.6 | Resistivity |
| D10 | 900 | 1+300 | iii | 0.4 | Seismic |
| D11 | 1+300 | 2+700 | iii | 1.4 | Resistivity |
| D12 | 2+700 | 3+100 | iii | 0.4 | Seismic |
| D13 | 3+100 | 5+300 | iii | 3.2 | Resistivity |
| D14 | 8+500 | 11+000 | iii | 2.5 | Resistivity |
| D15 | 12+000 | 13+000 | iii | 1 | Resistivity |
| D16 | 15+000 | 18+000 | iii | 3 | Resistivity |
| D17 | 20+000 | 21+000 | iii | 1 | Resistivity |
| D18 | 25+200 | 25+900 | iii | 0.7 | Resistivity |
| D19 | 29+500 | 30+700 | iii | 1.2 | Resistivity |
| D20 | 36+400 | 36+800 | iii | 0.4 | Seismic |
| D21 | 38+600 | 39+300 | iii | 0.7 | Resistivity |
| D22 | 39+300 | 39+700 | iii | 0.4 | Seismic |
| D23 | 39+800 | 41+300 | iii | 0.5 | Resistivity |
| D24 | 43+200 | 43+500 | iii | 0.3 | Seismic |
| D25 | 51+700 | 55+300 | iii | 3.6 | Seismic |
| D26 | 68+600 | 69+800 | iii | 0.2 | Seismic |
| D27 | 70+800 | 71+600 | iii | 0.8 | Resistivity |
| D28 | 90+700 | 91+300 | iii | 0.6 | Seismic |
| D29 | 91+800 | 92+600 | iii | 0.8 | Resistivity |
| D30 | 96+200 | 98+200 | iii | 2 | Resistivity |
| D31 | 1+000 | 1400 | ii | 0.4 | Seismic |
| D32 | 9+000 | 10+000 | ii | 1 | Seismic |
| D33 | 14+500 | 14+900 | ii | 0.4 | Seismic |
| D34 | 20+900 | 21+600 | ii | 0.7 | Seismic |
| D35 | 27+300 | 27+700 | ii | 0.4 | Seismic |
| D36 | 29+500 | 29+900 | ii | 0.4 | Seismic |
| D37 | 32+000 | 32+400 | ii | 0.4 | Seismic |
| D38 | 27+700 | 29+000 | ii | 1.3 | Resistivity |
| D39 | 62+500 | 64+000 | ii | 1.5 | Seismic |
| D40 | 71+000 | 71+700 | ii | 0.7 | Seismic |
| D41 | 73+000 | 73+400 | ii | 0.4 | Seismic |

# Introduction

Geophysical Survey presented within this report is part of Geotechnical Investigation Works in Connection with the Technical Assistance for the Preparation of Road Project Pipeline for the cohesion Fund Contract No.1: Package D.

Package D comprises delineation by geophysical means of depth to bedrock, bedrock profile as well as nature and extent of the overburden.

Volume D11 requirements: Resistivity Measurements within the area delineated by Task iii, Km 1.300 – 2.700 of projected motorway route.

Present report will describe data processing workflow and results within D11 area, as well as recommendation regarding expected nature and extent of overburden and bedrock as well as other detected sources.

Chargeability measurements were acquired as additional data and have been used to constrain interpretation within this report.

# Site overview

D11 Area is located between Babsa and Belint villages, south of Timis - Bega Chanel and east of Bega River.  Terrain morphology is flat, being covered by scattered but dense wild vegetation.

According to available geological information, within D11 Survey Area Quaternary fluvial deposits are expected, most likely comprising clay, sand and gravel.

# Fieldwork

In order to acquire a reliable resistivity data set, considering project requirements, pole-dipole array was used, being easy to implement and having proven results. Successive resistivity and chargeability measurements, at 20 meters spacing and 16 levels up to 40 meters depth, insured reliable information on the electric characteristics of the surveyed area.

# Workflow

To proceed to interpretation on the nature of the detected sources, resistivity values have been linked to the geological data using Rock Resistivity Tables like the one bellow:

| Material | Resistivity Range (Ωm) |
| --- | --- |
| Clay | 1 – 102 |
| Sand | 1 – 103 |
| Gravel | 10 - 104 |
| Sandstone | 1 – 108 |
| Dolomite | 102 - 104 |
| Limestone | 50 – 107 |
| Basalt | 10 – 107 |
| Gabbro | 103 – 106 |
| Granite | 102 – 106 |

Often the resistivity ranges overlap requiring additional data to pin-point the exact nature of the source. Variations in resistivity ranges are caused by moisture and general structural integrity of the detected source.

Measured resistivity values are subject to data inversion having as result an easily interpretable geophysical model of the surveyed area. Over this model, considering distribution of resistivity and chargeability, primary and secondary lithological lines were drawn in order to delineate intercepted sources/layers.

Primary lithological lines were placed especially on high gradient zones, thus representing high contrast limits.

Secondary lithological lines were placed in areas where more subtle changes in geophysical parameters distribution are visible. These lines represent a less precise delineation of sources.

# Data Interpretation

Given the above considerations, within D11 Area the following sources have been detected (see Annex 11/41 – a and Annex 11/41 – b):

| Source Type | Resistivity Signature | Chargeability Signature | Position / Dimensions | Expected source |
| --- | --- | --- | --- | --- |
| So | Inhomogeneous medium low values (<100 Ωm) | Very low values (<5 mV/v) | Quasi-horizontal continuous layered source, 2-6 meters thick | Soil + Clays ± Sand |
| SG | Medium-high values (100-400 Ωm) | Low values (5-10 mV/V) | Quasi-horizontal continuous layered source, ~20 meters thick | Sand and Gravel |
| Cy | Low to very low values (10-50 Ωm) | Inhomogeneous medium values, (10-30 mV/V) | Quasi-horizontal continuous layered source, at 20-25 m deep up to maximum investigation depth | Clay deposits ± Sand and Gravel Intercalations |
| SG? | Change in resistivity distribution | - | Possible existence of intercalations (low confidence degree) | Sand and Gravel |
| Cy? | Low to very low values (10-50 Ωm) | Very low values (<5 mV/v) | Possible lenticular clay type sediments (low confidence degree) | Clay |

# Conclusions and Recommendations

D11 survey line intercepted a three layer lithological succession likely comprising: soil and clays ± sand, sand and gravel and a predominant clay layer.

Resistivity as well as chargeability characteristic values for the near surface layer (source type So) indicate this as being overburden layer (expected to have weak structural integrity).

Resistivity Survey Project Coordinator

Eng. Filip ONESCU

Report and Drawings prepared by

Eng. Filip ONESCU

Verified by:

Eng. Horia NISTOR

Approved by:

Eng. Dan ONESCU