VOLUME D12

GEOPHYSICAL INVESTIGATION

SEISMIC SURVEY

| Volume | Km from | Km to | Task | Length | Survey Type |
| --- | --- | --- | --- | --- | --- |
| D01 | 800 | 1+300 | i | 0.5 | Seismic |
| D02 | 27+200 | 27+500 | i | 0.3 | Seismic |
| D03 | 28+400 | 28+600 | i | 0.2 | Seismic |
| D04 | 28+600 | 29+600 | i | 1 | Resistivity |
| D05 | 36+800 | 37+300 | i | 0.5 | Resistivity |
| D06 | 37+300 | 37+800 | i | 0.5 | Seismic |
| D07 | 37+800 | 38+900 | i | 1.2 | Resistivity |
| D08 | 10+200 | 10+700 | iii LLR | 0.5 | Seismic |
| D09 | 300 | 900 | iii | 0.6 | Resistivity |
| D10 | 900 | 1+300 | iii | 0.4 | Seismic |
| D11 | 1+300 | 2+700 | iii | 1.4 | Resistivity |
| D12 | 2+700 | 3+100 | iii | 0.4 | Seismic |
| D13 | 3+100 | 5+300 | iii | 3.2 | Resistivity |
| D14 | 8+500 | 11+000 | iii | 2.5 | Resistivity |
| D15 | 12+000 | 13+000 | iii | 1 | Resistivity |
| D16 | 15+000 | 18+000 | iii | 3 | Resistivity |
| D17 | 20+000 | 21+000 | iii | 1 | Resistivity |
| D18 | 25+200 | 25+900 | iii | 0.7 | Resistivity |
| D19 | 29+500 | 30+700 | iii | 1.2 | Resistivity |
| D20 | 36+400 | 36+800 | iii | 0.4 | Seismic |
| D21 | 38+600 | 39+300 | iii | 0.7 | Resistivity |
| D22 | 39+300 | 39+700 | iii | 0.4 | Seismic |
| D23 | 39+800 | 41+300 | iii | 0.5 | Resistivity |
| D24 | 43+200 | 43+500 | iii | 0.3 | Seismic |
| D25 | 51+700 | 55+300 | iii | 3.6 | Seismic |
| D26 | 68+600 | 69+800 | iii | 0.2 | Seismic |
| D27 | 70+800 | 71+600 | iii | 0.8 | Resistivity |
| D28 | 90+700 | 91+300 | iii | 0.6 | Seismic |
| D29 | 91+800 | 92+600 | iii | 0.8 | Resistivity |
| D30 | 96+200 | 98+200 | iii | 2 | Resistivity |
| D31 | 1+000 | 1400 | ii | 0.4 | Seismic |
| D32 | 9+000 | 10+000 | ii | 1 | Seismic |
| D33 | 14+500 | 14+900 | ii | 0.4 | Seismic |
| D34 | 20+900 | 21+600 | ii | 0.7 | Seismic |
| D35 | 27+300 | 27+700 | ii | 0.4 | Seismic |
| D36 | 29+500 | 29+900 | ii | 0.4 | Seismic |
| D37 | 32+000 | 32+400 | ii | 0.4 | Seismic |
| D38 | 27+700 | 29+000 | ii | 1.3 | Resistivity |
| D39 | 62+500 | 64+000 | ii | 1.5 | Seismic |
| D40 | 71+000 | 71+700 | ii | 0.7 | Seismic |
| D41 | 73+000 | 73+400 | ii | 0.4 | Seismic |

# Introduction

Geophysical Survey presented within this report is part of Geotechnical Investigation Works in Connection with the Technical Assistance for the Preparation of Road Project Pipeline for the cohesion Fund Contract No.1: Package D.

Package D comprises delineation by geophysical means of depth to bedrock, bedrock profile as well as nature and extent of the overburden.

Volume D12 requirements: Seismic Measurements within the area delineated by Task iii, Km 2.700 – 3.100 of projected motorway route.

Present report will describe data processing workflow and results within D12 area, as well as recommendation regarding expected nature and extent of overburden and bedrock as well as other detected sources.

# Site overview

D12 Area is located north of Belint crossing Timis - Bega Channel.  Terrain morphology is mostly flat, being covered by scattered vegetation.

According to available geological information, within D12 Survey Area Quaternary deposits are expected, most likely comprising clay, sand and gravel.

# Fieldwork

The recommended survey method, considering project requirements, is Seismic Refraction. This method is widely used, with proven results for delineating near surface lithological features.

At 2.5 spaced geophones and 11 shots per spread, the array used for seismic data acquisition insures high detail results. High signal to noise ratio is insured by stacking functions and level three CDP gather.

Marginal offset shots are included within plan seismic survey line representation. These shots, although part of the acquisition process, are not considered part of the Seismic Survey length (Seismic Velocity Section).

# Workflow

To proceed to interpretation on the nature of the detected sources, velocity values have been linked to the geological data using Material Velocity Tables like the one bellow:

| Material | Velocity Range (m/s) |
| --- | --- |
| Clay | 1000 – 2500 |
| Sand (dry, loose) | 200 – 1000 |
| Sand(water saturated) | 1500 - 2000 |
| Sand and Gravel | 400 - 2300 |
| Sandstone | 1400 – 4500 |
| Dolomite | 2500 - 6500 |
| Limestone | 2800 – 7000 |
| Basalt | 5500 – 6500 |
| Gabbros | 6400 – 7000 |
| Granite | 4600 – 6200 |

Often the velocity values range overlap requiring additional data to pin-point the exact nature of the source. Variations in velocity ranges are caused by moisture and general structural integrity of the detected source.

Measured velocity values are subject to data inversion having as result an easily interpretable geophysical model of the surveyed area. Over this model, considering distribution of the velocities, primary and secondary lithological lines were drawn in order to delineate intercepted sources/layers.

Secondary lithological lines were placed in areas where more subtle changes in geophysical parameters distribution are visible. These lines represent a less precise delineation of sources.

Primary lithological lines were placed especially on high gradient zones, thus representing high contrast limits.

Generally, for shallow seismic refraction, low velocity values can be linked to areas with weak structural integrity being considered as an indicator for potentially unstable deposits.

The result of seismic interpretation is a velocity section. These values, without additional data, describe seismic boundaries (refractors). The actual geological nature of the sources can not be estimated solely on absolute velocity values.

# Data Interpretation

Given the above considerations, within D12 Area the following sources have been detected (see Annex 12/41):

| Source Type | Velocity signature | Position / Dimensions | Expected source |
| --- | --- | --- | --- |
| So | Very low values (<400 m/s) | Quasi-horizontal continuous layered source, 1-2 meters thick | Very low velocity source (Soil, loose sediments) |
| S1 | High gradient 500 – 1500 m/s | Quasi-horizontal continuous layered source 2-5 meters thick | Low velocity source (unconsolidated and/or moist sediments) |
| S2 | Medium values (1500 – 2000 m/s) | Quasi-horizontal layered source, ~ 6 meter depth | Medium velocity source |

# Conclusions and Recommendations

The first source type (So - soil and loose sediments) has very low velocity values. This layer is expected to have structural instability.

Seismic Survey Project Coordinator

Eng. Serban NEGREA

Report and Drawings prepared by

Eng. Serban NEGREA

Verified by:

Eng. Horia NISTOR

Approved by:

Eng. Dan ONESCU