VOLUME D13

GEOPHYSICAL INVESTIGATION

RESISTIVITY SURVEY

| Volume | Km from | Km to | Task | Length | Survey Type |
| --- | --- | --- | --- | --- | --- |
| D01 | 800 | 1+300 | i | 0.5 | Seismic |
| D02 | 27+200 | 27+500 | i | 0.3 | Seismic |
| D03 | 28+400 | 28+600 | i | 0.2 | Seismic |
| D04 | 28+600 | 29+600 | i | 1 | Resistivity |
| D05 | 36+800 | 37+300 | i | 0.5 | Resistivity |
| D06 | 37+300 | 37+800 | i | 0.5 | Seismic |
| D07 | 37+800 | 38+900 | i | 1.2 | Resistivity |
| D08 | 10+200 | 10+700 | iii LLR | 0.5 | Seismic |
| D09 | 300 | 900 | iii | 0.6 | Resistivity |
| D10 | 900 | 1+300 | iii | 0.4 | Seismic |
| D11 | 1+300 | 2+700 | iii | 1.4 | Resistivity |
| D12 | 2+700 | 3+100 | iii | 0.4 | Seismic |
| D13 | 3+100 | 5+300 | iii | 3.2 | Resistivity |
| D14 | 8+500 | 11+000 | iii | 2.5 | Resistivity |
| D15 | 12+000 | 13+000 | iii | 1 | Resistivity |
| D16 | 15+000 | 18+000 | iii | 3 | Resistivity |
| D17 | 20+000 | 21+000 | iii | 1 | Resistivity |
| D18 | 25+200 | 25+900 | iii | 0.7 | Resistivity |
| D19 | 29+500 | 30+700 | iii | 1.2 | Resistivity |
| D20 | 36+400 | 36+800 | iii | 0.4 | Seismic |
| D21 | 38+600 | 39+300 | iii | 0.7 | Resistivity |
| D22 | 39+300 | 39+700 | iii | 0.4 | Seismic |
| D23 | 39+800 | 41+300 | iii | 0.5 | Resistivity |
| D24 | 43+200 | 43+500 | iii | 0.3 | Seismic |
| D25 | 51+700 | 55+300 | iii | 3.6 | Seismic |
| D26 | 68+600 | 69+800 | iii | 0.2 | Seismic |
| D27 | 70+800 | 71+600 | iii | 0.8 | Resistivity |
| D28 | 90+700 | 91+300 | iii | 0.6 | Seismic |
| D29 | 91+800 | 92+600 | iii | 0.8 | Resistivity |
| D30 | 96+200 | 98+200 | iii | 2 | Resistivity |
| D31 | 1+000 | 1400 | ii | 0.4 | Seismic |
| D32 | 9+000 | 10+000 | ii | 1 | Seismic |
| D33 | 14+500 | 14+900 | ii | 0.4 | Seismic |
| D34 | 20+900 | 21+600 | ii | 0.7 | Seismic |
| D35 | 27+300 | 27+700 | ii | 0.4 | Seismic |
| D36 | 29+500 | 29+900 | ii | 0.4 | Seismic |
| D37 | 32+000 | 32+400 | ii | 0.4 | Seismic |
| D38 | 27+700 | 29+000 | ii | 1.3 | Resistivity |
| D39 | 62+500 | 64+000 | ii | 1.5 | Seismic |
| D40 | 71+000 | 71+700 | ii | 0.7 | Seismic |
| D41 | 73+000 | 73+400 | ii | 0.4 | Seismic |

# Introduction

Geophysical Survey presented within this report is part of Geotechnical Investigation Works in Connection with the Technical Assistance for the Preparation of Road Project Pipeline for the cohesion Fund Contract No.1: Package D.

Package D comprises delineation by geophysical means of depth to bedrock, bedrock profile as well as nature and extent of the overburden.

Volume D13 requirements: Resistivity Measurements within the area delineated by Task iii, Km 3.100 – 5.300 of projected motorway route.

Present report will describe data processing workflow and results within D13 area, as well as recommendation regarding expected nature and extent of overburden and bedrock as well as other detected sources.

Chargeability measurements were acquired as additional data and have been used to constrain interpretation within this report.

# Site overview

D13 Area is located between Babsa and Belint villages, north of Timis - Bega Chanel.  Terrain morphology is flat, being covered by scattered wild vegetation.

According to available geological information, within D13 Survey Area Quaternary fluvial deposits are expected, most likely comprising clay, sand and gravel.

# Fieldwork

In order to acquire a reliable resistivity data set, considering project requirements, pole-dipole array was used, being easy to implement and having proven results. Successive resistivity and chargeability measurements, at 20 meters spacing and 16 levels up to 40 meters depth, insured reliable information on the electric characteristics of the surveyed area.

Due to the inability to measure over Babsa-Belint road the projected survey line could not be placed on the exact D13 Volume span and had to be moved slightly towards north-west.

# Workflow

To proceed to interpretation on the nature of the detected sources, resistivity values have been linked to the geological data using Rock Resistivity Tables like the one bellow:

| Material | Resistivity Range (Ωm) |
| --- | --- |
| Clay | 1 – 102 |
| Sand | 1 – 103 |
| Gravel | 10 - 104 |
| Sandstone | 1 – 108 |
| Dolomite | 102 - 104 |
| Limestone | 50 – 107 |
| Basalt | 10 – 107 |
| Gabbro | 103 – 106 |
| Granite | 102 – 106 |

Often the resistivity ranges overlap requiring additional data to pin-point the exact nature of the source. Variations in resistivity ranges are caused by moisture and general structural integrity of the detected source.

Measured resistivity values are subject to data inversion having as result an easily interpretable geophysical model of the surveyed area. Over this model, considering distribution of resistivity and chargeability, primary and secondary lithological lines were drawn in order to delineate intercepted sources/layers.

Primary lithological lines were placed especially on high gradient zones, thus representing high contrast limits.

Secondary lithological lines were placed in areas where more subtle changes in geophysical parameters distribution are visible. These lines represent a less precise delineation of sources.

# Data Interpretation

Given the above considerations, within D13 Area the following sources have been detected (see Annex 13/41 – a, Annex 13/41 – b and Annex 13/41 – c):

| Source Type | Resistivity Signature | Chargeability Signature | Position / Dimensions | Expected source |
| --- | --- | --- | --- | --- |
| So | Inhomogeneous medium low values (<100 Ωm) | Very low values (<5 mV/v) | Quasi-horizontal continuous layered source, 2-10 meters thick | Soil + Clays ± Sand |
| SG | Medium-high values (100-300 Ωm) | Low values (5-10 mV/V) | Quasi-horizontal continuous layered source, 20-25 meters thick | Sand and Gravel |
| Cy | Low to very low values (10-50 Ωm) | Medium values (10-30 mV/V) | Quasi-horizontal continuous layered source, at 20-30 m deep up to maximum investigation depth | Clay |
| ART | Abnormally low (<5 Ωm); irregular shape | Abnormally high(>80 mV/V); irregular shape | Buried artifact, likely punctual source despite anomalous sector shape | Buried pipe/ live electric wire |

# Conclusions and Recommendations

D13 survey line intercepted a three layer lithological succession likely comprising: soil and clays ± sand, sand and gravel and a predominant clay layer.

The ART source type has a strong signature in both chargeability and resistivity thus masking in its area of influence other electric sources. This source, even after inversion, has a wide irregular spread. In most of the cases point type pipe like sources tend to have this type of signature. Therefore, the position of this source has been determined considering the very top, high gradient limit of the anomalous sector (mainly on chargeability) as opposed to the highest value. Generally, such sources with high contrast of electrical properties are miscalculated by the inversion program, therefore the location for this source should be considered as a low confidence determination.

Resistivity as well as chargeability characteristic values for the near surface layer (source type So) indicate this as being overburden layer (expected to have weak structural integrity).

Resistivity Survey Project Coordinator

Eng. Filip ONESCU

Report and Drawings prepared by

Eng. Filip ONESCU

Verified by:

Eng. Horia NISTOR

Approved by:

Eng. Dan ONESCU