VOLUME D16

GEOPHYSICAL INVESTIGATION

RESISTIVITY SURVEY

| Volume | Km from | Km to | Task | Length | Survey Type |
| --- | --- | --- | --- | --- | --- |
| D01 | 800 | 1+300 | i | 0.5 | Seismic |
| D02 | 27+200 | 27+500 | i | 0.3 | Seismic |
| D03 | 28+400 | 28+600 | i | 0.2 | Seismic |
| D04 | 28+600 | 29+600 | i | 1 | Resistivity |
| D05 | 36+800 | 37+300 | i | 0.5 | Resistivity |
| D06 | 37+300 | 37+800 | i | 0.5 | Seismic |
| D07 | 37+800 | 38+900 | i | 1.2 | Resistivity |
| D08 | 10+200 | 10+700 | iii LLR | 0.5 | Seismic |
| D09 | 300 | 900 | iii | 0.6 | Resistivity |
| D10 | 900 | 1+300 | iii | 0.4 | Seismic |
| D11 | 1+300 | 2+700 | iii | 1.4 | Resistivity |
| D12 | 2+700 | 3+100 | iii | 0.4 | Seismic |
| D13 | 3+100 | 5+300 | iii | 3.2 | Resistivity |
| D14 | 8+500 | 11+000 | iii | 2.5 | Resistivity |
| D15 | 12+000 | 13+000 | iii | 1 | Resistivity |
| D16 | 15+000 | 18+000 | iii | 3 | Resistivity |
| D17 | 20+000 | 21+000 | iii | 1 | Resistivity |
| D18 | 25+200 | 25+900 | iii | 0.7 | Resistivity |
| D19 | 29+500 | 30+700 | iii | 1.2 | Resistivity |
| D20 | 36+400 | 36+800 | iii | 0.4 | Seismic |
| D21 | 38+600 | 39+300 | iii | 0.7 | Resistivity |
| D22 | 39+300 | 39+700 | iii | 0.4 | Seismic |
| D23 | 39+800 | 41+300 | iii | 0.5 | Resistivity |
| D24 | 43+200 | 43+500 | iii | 0.3 | Seismic |
| D25 | 51+700 | 55+300 | iii | 3.6 | Seismic |
| D26 | 68+600 | 69+800 | iii | 0.2 | Seismic |
| D27 | 70+800 | 71+600 | iii | 0.8 | Resistivity |
| D28 | 90+700 | 91+300 | iii | 0.6 | Seismic |
| D29 | 91+800 | 92+600 | iii | 0.8 | Resistivity |
| D30 | 96+200 | 98+200 | iii | 2 | Resistivity |
| D31 | 1+000 | 1400 | ii | 0.4 | Seismic |
| D32 | 9+000 | 10+000 | ii | 1 | Seismic |
| D33 | 14+500 | 14+900 | ii | 0.4 | Seismic |
| D34 | 20+900 | 21+600 | ii | 0.7 | Seismic |
| D35 | 27+300 | 27+700 | ii | 0.4 | Seismic |
| D36 | 29+500 | 29+900 | ii | 0.4 | Seismic |
| D37 | 32+000 | 32+400 | ii | 0.4 | Seismic |
| D38 | 27+700 | 29+000 | ii | 1.3 | Resistivity |
| D39 | 62+500 | 64+000 | ii | 1.5 | Seismic |
| D40 | 71+000 | 71+700 | ii | 0.7 | Seismic |
| D41 | 73+000 | 73+400 | ii | 0.4 | Seismic |

# Introduction

Geophysical Survey presented within this report is part of Geotechnical Investigation Works in Connection with the Technical Assistance for the Preparation of Road Project Pipeline for the cohesion Fund Contract No.1: Package D.

Package D comprises delineation by geophysical means of depth to bedrock, bedrock profile as well as nature and extent of the overburden.

Volume D16 requirements: Resistivity Measurements within the area delineated by Task iii, Km 15.000 – 18.000 of projected motorway route.

Present report will describe data processing workflow and results within D16 area, as well as recommendation regarding expected nature and extent of overburden and bedrock as well as other detected sources.

Chargeability measurements were acquired as additional data and have been used to constrain interpretation within this report.

# Site overview

D16 Area is located between Cutina and Cliciova villages, south of Bega River.  Terrain morphology is flat, being covered by scattered but dense wild vegetation and agriculture land.

According to available geological information, within D16 Survey Area Quaternary fluvial deposits are expected, most likely comprising clay, sand and gravel.

# Fieldwork

In order to acquire a reliable resistivity data set, considering project requirements, pole-dipole array was used, being easy to implement and having proven results. Successive resistivity and chargeability measurements, at 20 meters spacing and 16 levels up to 40 meters depth, insured reliable information on the electric characteristics of the surveyed area.

# Workflow

To proceed to interpretation on the nature of the detected sources, resistivity values have been linked to the geological data using Rock Resistivity Tables like the one bellow:

| Material | Resistivity Range (Ωm) |
| --- | --- |
| Clay | 1 – 102 |
| Sand | 1 – 103 |
| Gravel | 10 - 104 |
| Sandstone | 1 – 108 |
| Dolomite | 102 - 104 |
| Limestone | 50 – 107 |
| Basalt | 10 – 107 |
| Gabbro | 103 – 106 |
| Granite | 102 – 106 |

Often the resistivity ranges overlap requiring additional data to pin-point the exact nature of the source. Variations in resistivity ranges are caused by moisture and general structural integrity of the detected source.

Measured resistivity values are subject to data inversion having as result an easily interpretable geophysical model of the surveyed area. Over this model, considering distribution of resistivity and chargeability, primary and secondary lithological lines were drawn in order to delineate intercepted sources/layers.

Primary lithological lines were placed especially on high gradient zones, thus representing high contrast limits.

Secondary lithological lines were placed in areas where more subtle changes in geophysical parameters distribution are visible. These lines represent a less precise delineation of sources.

# Data Interpretation

Given the above considerations, within D16 Area the following sources have been detected (see Annex 16/41 – a, Annex 16/41 – b and Annex 16/41 – c):

| Source Type | Resistivity Signature | Chargeability Signature | Position / Dimensions | Expected source |
| --- | --- | --- | --- | --- |
| So | Inhomogeneous medium low values (<100 Ωm) | Very low values (<5 mV/v) | Quasi-horizontal continuous layered source, 0.5-5 meters thick | Soil + Clays ± Sand |
| SG | Inhomogeneous medium values (100-250 Ωm) | Low values (5-10 mV/V) | Quasi-horizontal continuous layered source, 5-15 meters thick | Sand and Gravel |
| Cy | Low to very low values (10-50 Ωm) | Medium to High values (10-50 mV/V) | Quasi-horizontal continuous layered source, at 20-30 m deep up to maximum investigation depth | Clay |
| SG? | Low to very low values (10-50 Ωm) | Very low values (<5 mV/v) | Intermittent layered source at the bottom of the section (low confidence degree) | Sand and Gravel |

# Conclusions and Recommendations

D16 survey line intercepted a three layer lithological succession likely comprising: soil and clays ± sand, sand and gravel and a predominant clay layer. A sand and gravel layer is intermittently visible at the bottom of the section.

Variation in resistivity distribution for the second detected layer (SG type) can be caused by different proportions of clay intercalations and the ratio gravel/sand. Generally electrical signature of intercalations present within a certain layer will be engulfed in the signature of the “host” layer thus decreasing or increasing observed resistivity.

The third, CY type layered source has a strong chargeability characteristic. Resistivity was used exclusively to delineate this layer. The cause for high chargeability is not obvious and further data is needed to clear up the source.

Resistivity as well as chargeability characteristic values for the near surface layer (source type So) indicate this as being overburden layer (expected to have weak structural integrity). Some portions of the second layer especially where resistivity values drop bellow 80 Ωm, can also be included in the overburden definition. The drops in resistivity can be caused by clay intercalations and/or high sand/gravel ratio.

Resistivity Survey Project Coordinator

Eng. Filip ONESCU

Report and Drawings prepared by

Eng. Filip ONESCU

Verified by:

Eng. Horia NISTOR

Approved by:

Eng. Dan ONESCU